LI_test_procedure

4/21/08   /gm   updated 4/16/09

testing SIT level 1 data for release OK at Berkeley site

1) using safari BROWSER,  ftp://stereo.ssl.berkeley.edu/prerelease

2) copy the files to a new folder in data/sit/sit_level_1_at_berkeley

3) in IDLDE, select the path to desktop/allidlcode/general -- the RELAUNCH IDLDE

4) .compile L1_verification

5) run L1_verification -- will prompt for s/c, folder etc.

6) program produces a new file in the same folder as the cdf files -- with 1 hr averages

7) read newfiles into kaleidagraph and spot check with 1-hr average stereo data

8) TURN OFF the  desktop/allidlcode/general path in IDLDE preferences/paths, since this

contains routines with same names other IDL routines and they are not compatible!!!

9) quit IDLDE

SPOT checks with Kaleidagraph:

Note:  L1 Berkeley files include empty boxes filed with NaN -- this table avoids them

| Sit-a |  | Sit-b |  |
| --- | --- | --- | --- |
| H #3 | H .394 | 3He #1 | 3He .190 |
| 4He #4 | 4He .137 | 4He #3 | 4He 0.0966 * |
| O #2 | O .067 | C #4 | C .095 |
|  |  | Fe # 1 | Fe .034 |

* this box had a significant efficiency chance sent to Peter S Aug 25, 2008;  affects whole mission

data set as of this writing (1/28/09);  2 other He boxes also changed, one on A, one additinal one on B;

see 8/25/08 notes for details.